# Multi-Rail Power Management Board

## for a SoC + Sensor Cluster

A Hardware Project Build Guide — Qualcomm Hardware Interview Portfolio

Prepared for: Rakshit Goel  |  B.E. EEE, BITS Pilani Hyderabad

This guide walks you, step by step, through designing, simulating, and documenting a multi-rail power management board — a compressed, interview-ready version of a real hardware bring-up project. Every step tells you exactly what to click, what to type, and what to check. Follow it top to bottom in one sitting (~2.5–3 hours).

# Table of Contents

# 0. Project Overview

You are designing a board that takes a single 5V input and produces three clean, correctly-sequenced power rails (1.2V, 1.8V, 3.3V) suitable for powering an SoC + camera sensor + IMU cluster — the same class of problem Qualcomm hardware teams solve for Snapdragon platforms.

## What you will produce by the end

- A written requirements spec (rail voltages, currents, sequencing order)

- A component-level Bill of Materials (BOM) with real part numbers

- An LTSpice simulation proving correct sequencing and acceptable transient/ripple behavior

- A KiCad schematic (professionally drawn, ERC-clean)

- A one-page written summary you can show in an interview or attach to your resume/portfolio

NOTE: Physically fabricating and bench-testing the board takes days (PCB fab + shipping) and is NOT part of this 3-hour session. This guide produces a fully simulated, schematic-level design — which is what most interviewers actually want to discuss anyway. Section 8 tells you exactly how to phrase this honestly.

## Time budget

| Phase | Time | Section |
| --- | --- | --- |
| Requirements + BOM | 25 min | Section 1 & 2 |
| LTSpice simulation | 60–70 min | Section 3 |
| KiCad schematic | 45–55 min | Section 4 |
| Optional PCB layout pass | 20 min (optional) | Section 5 |
| Write-up + packaging | 20 min | Section 6 |

# 1. Lock the Requirements (do not skip — this is your interview narrative)

Copy this spec as-is. It is realistic and defensible in an interview.

## 1.1 Rail table

| Rail | Voltage | Max Current | Powers | Regulator Type |
| --- | --- | --- | --- | --- |
| Input | 5V (USB) | 2A | Board input | — |
| Rail A | 1.2V | 500 mA | SoC core | LDO (low noise) |
| Rail B | 1.8V | 600 mA | I/O logic, IMU | Buck (sync.) |
| Rail C | 3.3V | 800 mA | Sensors, camera | Buck (sync.) |

## 1.2 Sequencing rule (write this down — you will be asked about it)

Real SoCs require the core rail to be stable before I/O rails power on, to avoid pin latch-up / undefined logic states. Your rule:

- t = 0 ms: Rail A (1.2V, core) enabled first

- t = 2 ms: Rail B (1.8V) enabled, only after Rail A has settled

- t = 4 ms: Rail C (3.3V) enabled, only after Rail B has settled

Power-down should happen in reverse order (3.3V off first, 1.2V off last) — mention this even if you don't simulate it, it shows you understand the full picture.

## 1.3 Protection features

- Reverse-polarity protection diode on the 5V input

- A P-MOSFET load switch on Rail C (3.3V) for hot-plug protection of the sensor cluster

- Test points (TP) on every rail and every enable line for bring-up debugging

# 2. Bill of Materials (BOM)

These are real, well-documented parts with freely available datasheets — use these exact part numbers so your LTSpice and KiCad steps below work without hunting for substitutes.

| Ref | Part | Function | Package |
| --- | --- | --- | --- |
| U1 | TI TPS62203 (or TPS62240) | 3.3V synchronous buck | SOT23-5 |
| U2 | TI TPS62240 | 1.8V synchronous buck | SOT23-5 / WSON |
| U3 | TI TLV700 series | 1.2V low-noise LDO | SOT23-5 |
| Q1 | AO3401 (or similar P-MOSFET) | Load switch on Rail C | SOT23-3 |
| D1 | SS34 Schottky diode | Reverse polarity protection | SMA/DO-214 |
| R/C network | Standard RC (10k / 1µF etc.) | Enable-pin sequencing delay | 0603 |
| C_in/C_out | 10µF / 22µF ceramic (X7R) | Input/output filter caps per regulator | 0805/1206 |

NOTE: Download each IC's datasheet PDF now (search "<part number> datasheet TI") — you'll need the recommended external component values (inductor, capacitors) from the "Application Information" section of each datasheet for both LTSpice and KiCad.

# 3. LTSpice Simulation — Step by Step

Goal: prove your sequencing timing and rail stability BEFORE drawing the schematic. This is the section that gives you real engineering evidence to show in an interview.

## 3.1 Get the tools and models (10 min)

- Open LTSpice (download free from analog.com/ltspice if not installed).

- For each regulator (TPS62203, TPS62240, TLV700), search "<part number> LTSpice model" or "<part number> PSpice model" on TI's product page → download the .lib/.asc/.mod file from the "Design & Development" or "Tools & Software" tab of that part's TI page.

- If a specific model is hard to find fast: don't get stuck. Use LTSpice's built-in generic behavioral shortcut instead — build each regulator as an ideal switch + feedback block (see 3.2 fallback) so you keep moving. You can swap in the real model later without losing your simulation structure.

- Place the downloaded .lib file in the same folder as your .asc project file, then add this line to your schematic: right-click canvas → "SPICE Directive" → type: .lib <filename>.lib

## 3.2 Build the circuit (25–30 min)

- File → New Schematic. Save as PowerRails.asc immediately.

- Place a 5V DC voltage source (component key "F2" or Edit → Component → voltage) as your input rail. Label its net "VIN_5V" (F4 → Label Net).

- Place the 3 regulator symbols (from the imported models) OR, for the fast fallback: use a Behavioral Voltage Source (component "bv") per rail, with an expression like: V = if(time > 0m, 1.2, 0)  for Rail A — this creates a clean step at the sequencing time without needing exact IC models.

- Wire VIN_5V → into each regulator's input pin. Wire each regulator's output to its own net: label them RAIL_A_1V2, RAIL_B_1V8, RAIL_C_3V3.

- Add the enable-pin RC delay: place a resistor (10k) + capacitor (1µF) in series from VIN_5V to each regulator's EN pin, sized so Rail A enables at 0ms, Rail B at ~2ms, Rail C at ~4ms. (Delay ≈ R×C × ln(Vin/(Vin-Vth)) — or simpler: just eyeball R/C ratios across the 3 rails so B's RC constant is roughly double A's, and C's is roughly double B's.)

- Add output capacitors: 22µF ceramic cap from each RAIL_x net to ground (matches typical datasheet recommendation).

- Add a resistive load on each output: 1 resistor per rail sized as V/I from your Section 1.1 table (e.g., 1.2V / 0.5A → 2.4Ω for Rail A) so the sim reflects a realistic load.

## 3.3 Run the sequencing simulation (10 min)

- Right-click canvas → "SPICE Directive" → type: .tran 20m  (runs a 20ms transient sim).

- Click the Run button (the running-man icon).

- On the resulting blank plot window, click each of the 3 rail nets (RAIL_A_1V2, RAIL_B_1V8, RAIL_C_3V3) in the schematic — LTSpice adds each as a trace on the same graph.

- You should see 3 step waveforms, staggered in time, each settling cleanly at its target voltage. This single plot is your key deliverable — screenshot it (Tools → Copy bitmap or just a screen capture).

- Verify: Rail A rises first and is stable before Rail B starts rising; Rail B is stable before Rail C starts. If they overlap incorrectly, increase the RC delay time constant on the later rail and re-run.

## 3.4 Run a load-step / ripple test (10–15 min)

- On the Rail C (3.3V) load resistor, replace it with a Behavioral or PULSE current source that steps from light load to full load (e.g., a PULSE(0.1 0.8 5m 1u 1u 5m 10m) current sink) to mimic a sensor suddenly waking up and drawing current.

- Re-run .tran and zoom into the RAIL_C_3V3 trace right at the step — note the dip (undershoot) and recovery time. Compare against typical datasheet ripple spec (usually ±3% of nominal is considered good).

- Screenshot this zoomed trace as your second deliverable — label it "Load transient response, Rail C."

NOTE: If your numbers look extreme (huge overshoot/undershoot), don't panic — it usually means output cap is too small or load step is unrealistically fast. Bump C_out to 47µF or slow the PULSE rise time to 10µs and re-run. Talking through this kind of tuning is itself good interview material.

# 4. KiCad Schematic — Step by Step

You already have this workflow from your Lenul Global internship — this section should move quickly.

## 4.1 Set up the project (5 min)

- Open KiCad → File → New Project → name it PowerRailsBoard → this creates the .kicad_pro, opens Schematic Editor automatically.

- In Schematic Editor: Preferences → Manage Symbol Libraries → confirm the default "Regulator_Switching", "Regulator_Linear", and "Device" libraries are enabled (they are, by default, in a fresh install).

## 4.2 Place and wire components (25–30 min)

- Place symbol ‘Regulator_Switching:TPS62203’ (or nearest match; if exact symbol missing, use a generic ‘Regulator_Switching’ block and rename its value field to your part number) — keyboard shortcut A to add symbol, search by typing the part name.

- Repeat for the 1.8V buck and the 1.2V LDO (search ‘Regulator_Linear’ library for the LDO symbol).

- Place a generic connector or voltage symbol for VIN_5V (Place → Add Power Symbol, or a simple label). Wire it through D1 (search ‘D_Schottky’ in the Device library) into all 3 regulator inputs.

- Place Q1 (search ‘Q_PMOS_GSD’ or similar in Device library) as the load switch on the 3.3V rail output, gate driven from a GPIO/enable net.

- Wire each regulator's EN pin back to your RC sequencing network — place 2 resistors and 2 capacitors (Device library: R, C) per your Section 3 delay values, in series from VIN to each EN pin.

- Add decoupling/output capacitors on every rail net per your BOM (values from Section 2).

- Place ‘Test Point’ symbols (search ‘TestPoint’ in Connector library) on every rail (RAIL_A, RAIL_B, RAIL_C) and every EN line — exactly like your Lenul Global pin-level documentation work.

- Label every net clearly using the Label tool (shortcut L): VIN_5V, EN_A, EN_B, EN_C, RAIL_A_1V2, RAIL_B_1V8, RAIL_C_3V3.

## 4.3 Annotate and check (10 min)

- Tools → Annotate Schematic → accept defaults → Annotate (auto-assigns U1, U2, R1, C1 etc. reference designators).

- Tools → Electrical Rules Checker (ERC) → Run. Fix any ‘unconnected pin’ or ‘power pin not driven’ warnings — usually means you forgot a ground or power symbol somewhere. Re-run until clean or only expected warnings remain.

- File → Export → Netlist, and File → Export → BOM — generate both. These two files are hard evidence of a complete, professional schematic — keep them.

## 4.4 Optional: quick PCB placement pass (~20 min, optional)

- Open PCB Editor (from the project window) → Update PCB from Schematic (F8 or Tools menu) to import all footprints.

- Roughly place the 3 regulator ICs spaced apart, input on the left edge, outputs flowing left-to-right toward Q1 and the output test points on the right edge — mirrors realistic power-flow layout thinking.

- Don't attempt a fully routed, DRC-clean board in this optional pass — a sensible placement with a couple of routed traces is enough to have a board image for your write-up. Full routing is future work, and it's fine to say so.

# 5. Package the Project (20 min)

Write a 1-page summary — this is what you'll actually show recruiters/interviewers. Use this structure:

| Section | Content |
| --- | --- |
| Problem | 1–2 sentences: a SoC + sensor cluster needs 3 clean, correctly sequenced rails from a single 5V input; incorrect sequencing risks pin latch-up. |
| Approach | Rail architecture table (Section 1.1) + sequencing rule (1.2) |
| Simulation | Embed the LTSpice sequencing plot + the load-transient plot, 2–3 sentences explaining what they show |
| Schematic | Screenshot of the KiCad schematic; mention ERC-clean, test points on every rail |
| Next steps | “Next step: fabricate a 2-layer PCB and validate against these simulated waveforms on the bench with a scope and e-load.” — honest and forward-looking |

NOTE: Keep the 'next steps' line — it's the difference between overselling an unfinished board and honestly presenting a scoped, simulation-validated design with a clear execution path. Interviewers respect the second framing.

# 6. How to Talk About This in an Interview

Anticipate these questions and have a one-line answer ready:

- “Why does core rail need to come up before I/O?” → Explain latch-up / undefined logic state risk if I/O is powered before the core logic that reads it.

- “Why an LDO for the 1.2V core instead of a buck?” → Lower output noise, important for sensitive digital core supplies, at the cost of efficiency — acceptable tradeoff at this current level.

- “What would you change for a fabricated board?” → Real component SPICE models validated against datasheet transient response curves, thermal analysis on the bucks under full load, and a 4-layer stack-up for better ground return paths.

- “How does this connect to your Lenul Global internship?” → Directly — you already modified a legacy PCB to add a dedicated 5V rail and isolated interface; this project applies the same power-rail and documentation discipline from a clean-sheet design.

# Appendix: Quick Checklist

- ☐ Requirements spec written (rails, currents, sequencing)

- ☐ BOM with real part numbers

- ☐ LTSpice sequencing plot (3 staggered rail step waveforms)

- ☐ LTSpice load-transient plot (Rail C step response)

- ☐ KiCad schematic, annotated, ERC-clean

- ☐ Netlist + BOM exported from KiCad

- ☐ (Optional) Rough PCB placement screenshot

- ☐ 1-page written summary with problem / approach / results / next steps

- ☐ Interview talking points rehearsed (Section 6)